杭州电子科技大学

网络空间安全学院

《网络仿真与模拟》实验任务书

2023.9.10

目录

1.	NS installation	1

2.	P2P network	1

2.1 First. CC	1

2.2 First. PY	3

3.	CSMA network	4

3.1	Second. CC	4

3.2 Second. PY	6

4.	Wifi Network	8

4.1 Third. CC	8

4.2 Third. PY	13

5.	Complex Experiments	16

- NS installation

See the file named “ns-3-installation0910.pdf”

Test your installation via running following command：

- P2P network

实验：

运行first.cc或first.py；

打印输出结果

解释输出结果中时延的来源。

2.1 First. CC

#include "ns3/applications-module.h"

#include "ns3/core-module.h"

#include "ns3/internet-module.h"

#include "ns3/network-module.h"

#include "ns3/point-to-point-module.h"

// Default Network Topology

//

//       10.1.1.0

// n0 -------------- n1

//    point-to-point

//

using namespace ns3;

NS_LOG_COMPONENT_DEFINE("FirstScriptExample");

int

main(int argc, char* argv[])

{

CommandLine cmd(__FILE__);

cmd.Parse(argc, argv);

Time::SetResolution(Time::NS);

LogComponentEnable("UdpEchoClientApplication", LOG_LEVEL_INFO);

LogComponentEnable("UdpEchoServerApplication", LOG_LEVEL_INFO);

NodeContainer nodes;

nodes.Create(2);

PointToPointHelper pointToPoint;

pointToPoint.SetDeviceAttribute("DataRate", StringValue("5Mbps"));

pointToPoint.SetChannelAttribute("Delay", StringValue("2ms"));

NetDeviceContainer devices;

devices = pointToPoint.Install(nodes);

InternetStackHelper stack;

stack.Install(nodes);

Ipv4AddressHelper address;

address.SetBase("10.1.1.0", "255.255.255.0");

Ipv4InterfaceContainer interfaces = address.Assign(devices);

UdpEchoServerHelper echoServer(9);

ApplicationContainer serverApps = echoServer.Install(nodes.Get(1));

serverApps.Start(Seconds(1.0));

serverApps.Stop(Seconds(10.0));

UdpEchoClientHelper echoClient(interfaces.GetAddress(1), 9);

echoClient.SetAttribute("MaxPackets", UintegerValue(1));

echoClient.SetAttribute("Interval", TimeValue(Seconds(1.0)));

echoClient.SetAttribute("PacketSize", UintegerValue(1024));

ApplicationContainer clientApps = echoClient.Install(nodes.Get(0));

clientApps.Start(Seconds(2.0));

clientApps.Stop(Seconds(10.0));

Simulator::Run();

Simulator::Destroy();

return 0;

}

2.2 First. PY

from ns import ns

# // Default Network Topology

# //

# //       10.1.1.0

# // n0 -------------- n1

# //    point-to-point

# //

ns.core.LogComponentEnable("UdpEchoClientApplication", ns.core.LOG_LEVEL_INFO)

ns.core.LogComponentEnable("UdpEchoServerApplication", ns.core.LOG_LEVEL_INFO)

nodes = ns.network.NodeContainer()

nodes.Create(2)

pointToPoint = ns.point_to_point.PointToPointHelper()

pointToPoint.SetDeviceAttribute("DataRate", ns.core.StringValue("5Mbps"))

pointToPoint.SetChannelAttribute("Delay", ns.core.StringValue("2ms"))

devices = pointToPoint.Install(nodes)

stack = ns.internet.InternetStackHelper()

stack.Install(nodes)

address = ns.internet.Ipv4AddressHelper()

address.SetBase(ns.network.Ipv4Address("10.1.1.0"),

ns.network.Ipv4Mask("255.255.255.0"))

interfaces = address.Assign(devices)

echoServer = ns.applications.UdpEchoServerHelper(9)

serverApps = echoServer.Install(nodes.Get(1))

serverApps.Start(ns.core.Seconds(1.0))

serverApps.Stop(ns.core.Seconds(10.0))

address = interfaces.GetAddress(1).ConvertTo()

echoClient = ns.applications.UdpEchoClientHelper(address, 9)

echoClient.SetAttribute("MaxPackets", ns.core.UintegerValue(1))

echoClient.SetAttribute("Interval", ns.core.TimeValue(ns.core.Seconds(1.0)))

echoClient.SetAttribute("PacketSize", ns.core.UintegerValue(1024))

clientApps = echoClient.Install(nodes.Get(0))

clientApps.Start(ns.core.Seconds(2.0))

clientApps.Stop(ns.core.Seconds(10.0))

ns.core.Simulator.Run()

ns.core.Simulator.Destroy()

- CSMA network

实验：

运行second.cc或second.py；

得到3个pcap追踪文件；

解析运行结果和3个追踪文件的内容。

  - Second. CC

#include "ns3/applications-module.h"

#include "ns3/core-module.h"

#include "ns3/csma-module.h"

#include "ns3/internet-module.h"

#include "ns3/ipv4-global-routing-helper.h"

#include "ns3/network-module.h"

#include "ns3/point-to-point-module.h"

// Default Network Topology

//

//       10.1.1.0

// n0 -------------- n1   n2   n3   n4

//    point-to-point  |    |    |    |

//                    ================

//                      LAN 10.1.2.0

using namespace ns3;

NS_LOG_COMPONENT_DEFINE("SecondScriptExample");

int

main(int argc, char* argv[])

{

bool verbose = true;

uint32_t nCsma = 3;

CommandLine cmd(__FILE__);

cmd.AddValue("nCsma", "Number of \"extra\" CSMA nodes/devices", nCsma);

cmd.AddValue("verbose", "Tell echo applications to log if true", verbose);

cmd.Parse(argc, argv);

if (verbose)

{

LogComponentEnable("UdpEchoClientApplication", LOG_LEVEL_INFO);

LogComponentEnable("UdpEchoServerApplication", LOG_LEVEL_INFO);

}

nCsma = nCsma == 0 ? 1 : nCsma;

NodeContainer p2pNodes;

p2pNodes.Create(2);

NodeContainer csmaNodes;

csmaNodes.Add(p2pNodes.Get(1));

csmaNodes.Create(nCsma);

PointToPointHelper pointToPoint;

pointToPoint.SetDeviceAttribute("DataRate", StringValue("5Mbps"));

pointToPoint.SetChannelAttribute("Delay", StringValue("2ms"));

NetDeviceContainer p2pDevices;

p2pDevices = pointToPoint.Install(p2pNodes);

CsmaHelper csma;

csma.SetChannelAttribute("DataRate", StringValue("100Mbps"));

csma.SetChannelAttribute("Delay", TimeValue(NanoSeconds(6560)));

NetDeviceContainer csmaDevices;

csmaDevices = csma.Install(csmaNodes);

InternetStackHelper stack;

stack.Install(p2pNodes.Get(0));

stack.Install(csmaNodes);

Ipv4AddressHelper address;

address.SetBase("10.1.1.0", "255.255.255.0");

Ipv4InterfaceContainer p2pInterfaces;

p2pInterfaces = address.Assign(p2pDevices);

address.SetBase("10.1.2.0", "255.255.255.0");

Ipv4InterfaceContainer csmaInterfaces;

csmaInterfaces = address.Assign(csmaDevices);

UdpEchoServerHelper echoServer(9);

ApplicationContainer serverApps = echoServer.Install(csmaNodes.Get(nCsma));

serverApps.Start(Seconds(1.0));

serverApps.Stop(Seconds(10.0));

UdpEchoClientHelper echoClient(csmaInterfaces.GetAddress(nCsma), 9);

echoClient.SetAttribute("MaxPackets", UintegerValue(1));

echoClient.SetAttribute("Interval", TimeValue(Seconds(1.0)));

echoClient.SetAttribute("PacketSize", UintegerValue(1024));

ApplicationContainer clientApps = echoClient.Install(p2pNodes.Get(0));

clientApps.Start(Seconds(2.0));

clientApps.Stop(Seconds(10.0));

Ipv4GlobalRoutingHelper::PopulateRoutingTables();

pointToPoint.EnablePcapAll("second");

csma.EnablePcap("second", csmaDevices.Get(1), true);

Simulator::Run();

Simulator::Destroy();

return 0;

}

3.2 Second. PY

from ns import ns

import sys

# // Default Network Topology

# //

# //       10.1.1.0

# // n0 -------------- n1   n2   n3   n4

# //    point-to-point  |    |    |    |

# //                    ================

# //                      LAN 10.1.2.0

from ctypes import c_int, c_bool

nCsma = c_int(3)

verbose = c_bool(True)

cmd = ns.CommandLine(__file__)

cmd.AddValue("nCsma", "Number of extra CSMA nodes/devices", nCsma)

cmd.AddValue("verbose", "Tell echo applications to log if true", verbose)

cmd.Parse(sys.argv)

if verbose.value:

ns.core.LogComponentEnable("UdpEchoClientApplication", ns.core.LOG_LEVEL_INFO)

ns.core.LogComponentEnable("UdpEchoServerApplication", ns.core.LOG_LEVEL_INFO)

nCsma.value = 1 if nCsma.value == 0 else nCsma.value

p2pNodes = ns.network.NodeContainer()

p2pNodes.Create(2)

csmaNodes = ns.network.NodeContainer()

csmaNodes.Add(p2pNodes.Get(1))

csmaNodes.Create(nCsma.value)

pointToPoint = ns.point_to_point.PointToPointHelper()

pointToPoint.SetDeviceAttribute("DataRate", ns.core.StringValue("5Mbps"))

pointToPoint.SetChannelAttribute("Delay", ns.core.StringValue("2ms"))

p2pDevices = pointToPoint.Install(p2pNodes)

csma = ns.csma.CsmaHelper()

csma.SetChannelAttribute("DataRate", ns.core.StringValue("100Mbps"))

csma.SetChannelAttribute("Delay", ns.core.TimeValue(ns.core.NanoSeconds(6560)))

csmaDevices = csma.Install(csmaNodes)

stack = ns.internet.InternetStackHelper()

stack.Install(p2pNodes.Get(0))

stack.Install(csmaNodes)

address = ns.internet.Ipv4AddressHelper()

address.SetBase(ns.network.Ipv4Address("10.1.1.0"), ns.network.Ipv4Mask("255.255.255.0"))

p2pInterfaces = address.Assign(p2pDevices)

address.SetBase(ns.network.Ipv4Address("10.1.2.0"), ns.network.Ipv4Mask("255.255.255.0"))

csmaInterfaces = address.Assign(csmaDevices)

echoServer = ns.applications.UdpEchoServerHelper(9)

serverApps = echoServer.Install(csmaNodes.Get(nCsma.value))

serverApps.Start(ns.core.Seconds(1.0))

serverApps.Stop(ns.core.Seconds(10.0))

echoClient = ns.applications.UdpEchoClientHelper(csmaInterfaces.GetAddress(nCsma.value).ConvertTo(), 9)

echoClient.SetAttribute("MaxPackets", ns.core.UintegerValue(1))

echoClient.SetAttribute("Interval", ns.core.TimeValue(ns.core.Seconds (1.0)))

echoClient.SetAttribute("PacketSize", ns.core.UintegerValue(1024))

clientApps = echoClient.Install(p2pNodes.Get(0))

clientApps.Start(ns.core.Seconds(2.0))

clientApps.Stop(ns.core.Seconds(10.0))

ns.internet.Ipv4GlobalRoutingHelper.PopulateRoutingTables()

pointToPoint.EnablePcapAll("second")

csma.EnablePcap ("second", csmaDevices.Get (1), True)

ns.core.Simulator.Run()

ns.core.Simulator.Destroy()

- Wifi Network

实验：

运行third.cc或third.py；

得到3个pcap追踪文件；

解析运行结果和3个追踪文件的内容。

4.1 Third. CC

#include "ns3/applications-module.h"

#include "ns3/core-module.h"

#include "ns3/csma-module.h"

#include "ns3/internet-module.h"

#include "ns3/mobility-module.h"

#include "ns3/network-module.h"

#include "ns3/point-to-point-module.h"

#include "ns3/ssid.h"

#include "ns3/yans-wifi-helper.h"

// Default Network Topology

//

//   Wifi 10.1.3.0

//                 AP

//  *    *    *    *

//  |    |    |    |    10.1.1.0

// n5   n6   n7   n0 -------------- n1   n2   n3   n4

//                   point-to-point  |    |    |    |

//                                   ================

//                                     LAN 10.1.2.0

using namespace ns3;

NS_LOG_COMPONENT_DEFINE("ThirdScriptExample");

int

main(int argc, char* argv[])

{

bool verbose = true;

uint32_t nCsma = 3;

uint32_t nWifi = 3;

bool tracing = false;

CommandLine cmd(__FILE__);

cmd.AddValue("nCsma", "Number of \"extra\" CSMA nodes/devices", nCsma);

cmd.AddValue("nWifi", "Number of wifi STA devices", nWifi);

cmd.AddValue("verbose", "Tell echo applications to log if true", verbose);

cmd.AddValue("tracing", "Enable pcap tracing", tracing);

cmd.Parse(argc, argv);

// The underlying restriction of 18 is due to the grid position

// allocator's configuration; the grid layout will exceed the

// bounding box if more than 18 nodes are provided.

if (nWifi > 18)

{

std::cout << "nWifi should be 18 or less; otherwise grid layout exceeds the bounding box"

<< std::endl;

return 1;

}

if (verbose)

{

LogComponentEnable("UdpEchoClientApplication", LOG_LEVEL_INFO);

LogComponentEnable("UdpEchoServerApplication", LOG_LEVEL_INFO);

}

NodeContainer p2pNodes;

p2pNodes.Create(2);

PointToPointHelper pointToPoint;

pointToPoint.SetDeviceAttribute("DataRate", StringValue("5Mbps"));

pointToPoint.SetChannelAttribute("Delay", StringValue("2ms"));

NetDeviceContainer p2pDevices;

p2pDevices = pointToPoint.Install(p2pNodes);

NodeContainer csmaNodes;

csmaNodes.Add(p2pNodes.Get(1));

csmaNodes.Create(nCsma);

CsmaHelper csma;

csma.SetChannelAttribute("DataRate", StringValue("100Mbps"));

csma.SetChannelAttribute("Delay", TimeValue(NanoSeconds(6560)));

NetDeviceContainer csmaDevices;

csmaDevices = csma.Install(csmaNodes);

NodeContainer wifiStaNodes;

wifiStaNodes.Create(nWifi);

NodeContainer wifiApNode = p2pNodes.Get(0);

YansWifiChannelHelper channel = YansWifiChannelHelper::Default();

YansWifiPhyHelper phy;

phy.SetChannel(channel.Create());

WifiMacHelper mac;

Ssid ssid = Ssid("ns-3-ssid");

WifiHelper wifi;

NetDeviceContainer staDevices;

mac.SetType("ns3::StaWifiMac", "Ssid", SsidValue(ssid), "ActiveProbing", BooleanValue(false));

staDevices = wifi.Install(phy, mac, wifiStaNodes);

NetDeviceContainer apDevices;

mac.SetType("ns3::ApWifiMac", "Ssid", SsidValue(ssid));

apDevices = wifi.Install(phy, mac, wifiApNode);

MobilityHelper mobility;

mobility.SetPositionAllocator("ns3::GridPositionAllocator",

"MinX",

DoubleValue(0.0),

"MinY",

DoubleValue(0.0),

"DeltaX",

DoubleValue(5.0),

"DeltaY",

DoubleValue(10.0),

"GridWidth",

UintegerValue(3),

"LayoutType",

StringValue("RowFirst"));

mobility.SetMobilityModel("ns3::RandomWalk2dMobilityModel",

"Bounds",

RectangleValue(Rectangle(-50, 50, -50, 50)));

mobility.Install(wifiStaNodes);

mobility.SetMobilityModel("ns3::ConstantPositionMobilityModel");

mobility.Install(wifiApNode);

InternetStackHelper stack;

stack.Install(csmaNodes);

stack.Install(wifiApNode);

stack.Install(wifiStaNodes);

Ipv4AddressHelper address;

address.SetBase("10.1.1.0", "255.255.255.0");

Ipv4InterfaceContainer p2pInterfaces;

p2pInterfaces = address.Assign(p2pDevices);

address.SetBase("10.1.2.0", "255.255.255.0");

Ipv4InterfaceContainer csmaInterfaces;

csmaInterfaces = address.Assign(csmaDevices);

address.SetBase("10.1.3.0", "255.255.255.0");

address.Assign(staDevices);

address.Assign(apDevices);

UdpEchoServerHelper echoServer(9);

ApplicationContainer serverApps = echoServer.Install(csmaNodes.Get(nCsma));

serverApps.Start(Seconds(1.0));

serverApps.Stop(Seconds(10.0));

UdpEchoClientHelper echoClient(csmaInterfaces.GetAddress(nCsma), 9);

echoClient.SetAttribute("MaxPackets", UintegerValue(1));

echoClient.SetAttribute("Interval", TimeValue(Seconds(1.0)));

echoClient.SetAttribute("PacketSize", UintegerValue(1024));

ApplicationContainer clientApps = echoClient.Install(wifiStaNodes.Get(nWifi - 1));

clientApps.Start(Seconds(2.0));

clientApps.Stop(Seconds(10.0));

Ipv4GlobalRoutingHelper::PopulateRoutingTables();

Simulator::Stop(Seconds(10.0));

if (tracing)

{

phy.SetPcapDataLinkType(WifiPhyHelper::DLT_IEEE802_11_RADIO);

pointToPoint.EnablePcapAll("third");

phy.EnablePcap("third", apDevices.Get(0));

csma.EnablePcap("third", csmaDevices.Get(0), true);

}

Simulator::Run();

Simulator::Destroy();

return 0;

}

4.2 Third. PY

from ns import ns

import sys

# // Default Network Topology

# //

# //   Wifi 10.1.3.0

# //                 AP

# //  *    *    *    *

# //  |    |    |    |    10.1.1.0

# // n5   n6   n7   n0 -------------- n1   n2   n3   n4

# //                   point-to-point  |    |    |    |

# //                                   ================

# //                                     LAN 10.1.2.0

from ctypes import c_bool, c_int

nCsma = c_int(3)

verbose = c_bool(True)

nWifi = c_int(3)

tracing = c_bool(False)

cmd = ns.CommandLine(__file__)

cmd.AddValue("nCsma", "Number of extra CSMA nodes/devices", nCsma)

cmd.AddValue("nWifi", "Number of wifi STA devices", nWifi)

cmd.AddValue("verbose", "Tell echo applications to log if true", verbose)

cmd.AddValue("tracing", "Enable pcap tracing", tracing)

cmd.Parse(sys.argv)

# The underlying restriction of 18 is due to the grid position

# allocator's configuration; the grid layout will exceed the

# bounding box if more than 18 nodes are provided.

if nWifi.value > 18:

print("nWifi should be 18 or less; otherwise grid layout exceeds the bounding box")

sys.exit(1)

if verbose.value:

ns.core.LogComponentEnable("UdpEchoClientApplication", ns.core.LOG_LEVEL_INFO)

ns.core.LogComponentEnable("UdpEchoServerApplication", ns.core.LOG_LEVEL_INFO)

p2pNodes = ns.network.NodeContainer()

p2pNodes.Create(2)

pointToPoint = ns.point_to_point.PointToPointHelper()

pointToPoint.SetDeviceAttribute("DataRate", ns.core.StringValue("5Mbps"))

pointToPoint.SetChannelAttribute("Delay", ns.core.StringValue("2ms"))

p2pDevices = pointToPoint.Install(p2pNodes)

csmaNodes = ns.network.NodeContainer()

csmaNodes.Add(p2pNodes.Get(1))

csmaNodes.Create(nCsma.value)

csma = ns.csma.CsmaHelper()

csma.SetChannelAttribute("DataRate", ns.core.StringValue("100Mbps"))

csma.SetChannelAttribute("Delay", ns.core.TimeValue(ns.core.NanoSeconds(6560)))

csmaDevices = csma.Install(csmaNodes)

wifiStaNodes = ns.network.NodeContainer()

wifiStaNodes.Create(nWifi.value)

wifiApNode = p2pNodes.Get(0)

channel = ns.wifi.YansWifiChannelHelper.Default()

phy = ns.wifi.YansWifiPhyHelper()

phy.SetChannel(channel.Create())

mac = ns.wifi.WifiMacHelper()

ssid = ns.wifi.Ssid ("ns-3-ssid")

wifi = ns.wifi.WifiHelper()

mac.SetType ("ns3::StaWifiMac", "Ssid", ns.wifi.SsidValue(ssid), "ActiveProbing", ns.core.BooleanValue(False))

staDevices = wifi.Install(phy, mac, wifiStaNodes)

mac.SetType("ns3::ApWifiMac","Ssid", ns.wifi.SsidValue (ssid))

apDevices = wifi.Install(phy, mac, wifiApNode)

mobility = ns.mobility.MobilityHelper()

mobility.SetPositionAllocator("ns3::GridPositionAllocator", "MinX", ns.core.DoubleValue(0.0),

"MinY", ns.core.DoubleValue (0.0), "DeltaX", ns.core.DoubleValue(5.0), "DeltaY", ns.core.DoubleValue(10.0),

"GridWidth", ns.core.UintegerValue(3), "LayoutType", ns.core.StringValue("RowFirst"))

mobility.SetMobilityModel ("ns3::RandomWalk2dMobilityModel", "Bounds", ns.mobility.RectangleValue(ns.mobility.Rectangle (-50, 50, -50, 50)))

mobility.Install(wifiStaNodes)

mobility.SetMobilityModel("ns3::ConstantPositionMobilityModel")

mobility.Install(wifiApNode)

stack = ns.internet.InternetStackHelper()

stack.Install(csmaNodes)

stack.Install(wifiApNode)

stack.Install(wifiStaNodes)

address = ns.internet.Ipv4AddressHelper()

address.SetBase(ns.network.Ipv4Address("10.1.1.0"), ns.network.Ipv4Mask("255.255.255.0"))

p2pInterfaces = address.Assign(p2pDevices)

address.SetBase(ns.network.Ipv4Address("10.1.2.0"), ns.network.Ipv4Mask("255.255.255.0"))

csmaInterfaces = address.Assign(csmaDevices)

address.SetBase(ns.network.Ipv4Address("10.1.3.0"), ns.network.Ipv4Mask("255.255.255.0"))

address.Assign(staDevices)

address.Assign(apDevices)

echoServer = ns.applications.UdpEchoServerHelper(9)

serverApps = echoServer.Install(csmaNodes.Get(nCsma.value))

serverApps.Start(ns.core.Seconds(1.0))

serverApps.Stop(ns.core.Seconds(10.0))

echoClient = ns.applications.UdpEchoClientHelper(csmaInterfaces.GetAddress(nCsma.value).ConvertTo(), 9)

echoClient.SetAttribute("MaxPackets", ns.core.UintegerValue(1))

echoClient.SetAttribute("Interval", ns.core.TimeValue(ns.core.Seconds (1.0)))

echoClient.SetAttribute("PacketSize", ns.core.UintegerValue(1024))

clientApps = echoClient.Install(wifiStaNodes.Get (nWifi.value - 1))

clientApps.Start(ns.core.Seconds(2.0))

clientApps.Stop(ns.core.Seconds(10.0))

ns.internet.Ipv4GlobalRoutingHelper.PopulateRoutingTables()

ns.core.Simulator.Stop(ns.core.Seconds(10.0))

if tracing.value:

phy.SetPcapDataLinkType(phy.DLT_IEEE802_11_RADIO)

pointToPoint.EnablePcapAll ("third")

phy.EnablePcap ("third", apDevices.Get (0))

csma.EnablePcap ("third", csmaDevices.Get (0), True)

ns.core.Simulator.Run()

ns.core.Simulator.Destroy()

- Complex Experiments

Objectives：Simulate a datacenter network using ns3. Experiment two traffic patterns. Examine the network. Improve the network.

Topology：

Descriptions：

（1）8 servers: n1~8; 4 ToR switches: t1~4; 2 aggregation switches: a1~2; 1 core switch: c1

（2）The network is partitioned into two clusters

（3）The links connecting to c1 are PPP, or the other networks are Ethernets.

（4）All the end-end delays on the networks are 500ns.

（5）IP address assignment is shown on the topology.

（6）All the switches behaves like OSPF routers.

Traffic Patterns:

(1) Pattern 1: inter-cluster traffic. For example, 1-5, 6-2, 3-7, 8-4. Each server communicates using TCP with another server that comes from different cluster

(2) Pattern 2: many-to-one traffic. Select one server as the sink, and all the other servers communicate to it.

Requirements:

Simulate the two patterns separately, obtain the throughput that the network can achieve, and find out the network bottleneck, how to improve the network.